/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : RS485通信终极修复版 - 解决接收方死锁

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "tim.h"

#include "usart.h"

#include "gpio.h"

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

typedef enum {STATE_OFF=0, STATE_RED, STATE_BLUE} LightState_t;

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

// 设备ID：编译时修改 (1,2,3,4)

#define MY_ID 2

// 系统参数

#define PWM_MAX 63999

#define FRAME_START 0xAA

#define BROADCAST_ADDR 0xFF

#define CMD_SET_STATE 0x01

#define DEBOUNCE_MS 20

#define FRAME_SIZE 4

// 调试引脚定义

#define DEBUG_TX_PIN GPIO_PIN_2  // PA2 - 发送指示

#define DEBUG_RX_PIN GPIO_PIN_3  // PA3 - 接收指示

// 按键宏定义

#ifndef KEY_Pin

#define KEY_Pin GPIO_PIN_0

#define KEY_GPIO_Port GPIOA

#endif

#ifndef SET_Pin

#define SET_Pin GPIO_PIN_1

#define SET_GPIO_Port GPIOA

#endif

/* USER CODE END PD */

/* Private variables ---------------------------------------------------------*/

/* USER CODE BEGIN PV */

// 全局变量

volatile LightState_t state = STATE_RED;

// RS485接收相关 - 简化设计

static uint8_t rx_byte;

static uint8_t rx_buf[FRAME_SIZE];

static uint8_t rx_index = 0;

static uint8_t expecting_frame = 0;  // 0=等待帧头, 1=接收数据

// 事件标志

volatile uint8_t frame_received = 0;

volatile uint8_t need_blink = 0;

// 调试计数

volatile uint32_t tx_count = 0;

volatile uint32_t rx_count = 0;

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

/* USER CODE BEGIN PFP */

void UpdateLEDs(void);

void SendState(LightState_t s);

void ProcessReceivedFrame(void);

void UART_StartReceive(void);

/* USER CODE END PFP */

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_TIM1_Init();

MX_USART1_UART_Init();

/* USER CODE BEGIN 2 */

// 初始化调试引脚

__HAL_RCC_GPIOA_CLK_ENABLE();

GPIO_InitTypeDef GPIO_InitStruct = {0};

GPIO_InitStruct.Pin = DEBUG_TX_PIN | DEBUG_RX_PIN;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

HAL_GPIO_WritePin(GPIOA, DEBUG_TX_PIN, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOA, DEBUG_RX_PIN, GPIO_PIN_RESET);

// 启动PWM

HAL_TIM_PWM_Start(&htim1, TIM_CHANNEL_1);

HAL_TIM_PWM_Start(&htim1, TIM_CHANNEL_4);

// 设置初始状态

UpdateLEDs();

// 启动串口接收

UART_StartReceive();

// RS485初始为接收模式

HAL_GPIO_WritePin(SET_GPIO_Port, SET_Pin, GPIO_PIN_RESET);

// 按键状态跟踪变量

uint8_t last_key_state = 1;  // 1=释放, 0=按下

uint32_t key_press_time = 0;

uint8_t key_processed = 0;   // 按键已处理标志

// 程序运行指示

for(int i=0; i<3; i++)

{

HAL_GPIO_TogglePin(GPIOA, DEBUG_TX_PIN);

HAL_Delay(100);

}

HAL_GPIO_WritePin(GPIOA, DEBUG_TX_PIN, GPIO_PIN_RESET);

/* USER CODE END 2 */

/* Infinite loop */

while(1)

{

// 1. 处理接收到的数据帧

if(frame_received)

{

frame_received = 0;

ProcessReceivedFrame();

}

// 2. 处理LED闪烁指示

if(need_blink)

{

need_blink = 0;

// 保存当前亮度

uint16_t saved_red = __HAL_TIM_GET_COMPARE(&htim1, TIM_CHANNEL_1);

uint16_t saved_blue = __HAL_TIM_GET_COMPARE(&htim1, TIM_CHANNEL_4);

// 闪烁

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0);

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_4, 0);

HAL_Delay(20);

// 恢复

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, saved_red);

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_4, saved_blue);

}

// 3. 轮询检测按键

uint8_t current_key_state = HAL_GPIO_ReadPin(KEY_GPIO_Port, KEY_Pin);

// 检测下降沿（按键按下）

if(last_key_state == 1 && current_key_state == 0)

{

key_press_time = HAL_GetTick();

key_processed = 0;  // 重置处理标志

}

// 检测上升沿（按键释放）且按下时间>20ms

if(last_key_state == 0 && current_key_state == 1)

{

if(!key_processed && (HAL_GetTick() - key_press_time) > DEBOUNCE_MS)

{

// 按键有效，切换状态

state = (state + 1) % 3;

UpdateLEDs();

// 发送状态到总线

SendState(state);

// 标记已处理

key_processed = 1;

// 防连按延时

HAL_Delay(200);

}

}

// 更新按键状态

last_key_state = current_key_state;

// 4. 程序运行指示 - DEBUG_TX_PIN每3秒闪烁一次

static uint32_t last_blink_time = 0;

if(HAL_GetTick() - last_blink_time > 3000)

{

last_blink_time = HAL_GetTick();

HAL_GPIO_TogglePin(GPIOA, DEBUG_TX_PIN);

}

// 短暂延时

HAL_Delay(1);

}

}

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

HAL_PWREx_ControlVoltageScaling(PWR_REGULATOR_VOLTAGE_SCALE1);

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSIDiv = RCC_HSI_DIV1;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

RCC_OscInitStruct.PLL.PLLM = RCC_PLLM_DIV1;

RCC_OscInitStruct.PLL.PLLN = 8;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLR = RCC_PLLR_DIV2;

if(HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK|RCC_CLOCKTYPE_PCLK1;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

if(HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_2) != HAL_OK)

{

Error_Handler();

}

}

/* USER CODE BEGIN 4 */

/**

* @brief 串口接收回调 - 简化版，避免复杂状态机

* @param huart: UART句柄

* @retval None

*/

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart)

{

if(huart->Instance == USART1)

{

// 接收指示

HAL_GPIO_WritePin(GPIOA, DEBUG_RX_PIN, GPIO_PIN_SET);

if(rx_byte == FRAME_START)

{

// 收到帧头，开始接收

expecting_frame = 1;

rx_index = 0;

rx_buf[rx_index++] = rx_byte;

}

else if(expecting_frame && rx_index < FRAME_SIZE)

{

// 接收数据字节

rx_buf[rx_index++] = rx_byte;

if(rx_index >= FRAME_SIZE)

{

// 收到完整帧

expecting_frame = 0;

frame_received = 1;  // 通知主循环处理

}

}

else

{

// 未同步到有效帧，重置

expecting_frame = 0;

rx_index = 0;

}

// 接收指示关闭

HAL_GPIO_WritePin(GPIOA, DEBUG_RX_PIN, GPIO_PIN_RESET);

// 重新启动接收 - 必须确保执行

HAL_UART_Receive_IT(&huart1, &rx_byte, 1);

}

}

/**

* @brief 启动串口接收

* @retval None

*/

void UART_StartReceive(void)

{

expecting_frame = 0;

rx_index = 0;

frame_received = 0;

HAL_UART_Receive_IT(&huart1, &rx_byte, 1);

}

/**

* @brief 更新LED状态

* @retval None

*/

void UpdateLEDs(void)

{

switch(state)

{

case STATE_OFF:

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0);

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_4, 0);

break;

case STATE_RED:

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, PWM_MAX);

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_4, 0);

break;

case STATE_BLUE:

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0);

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_4, PWM_MAX);

break;

}

}

/**

* @brief 发送状态到RS485总线

* @param s: 要发送的状态

* @retval None

*/

void SendState(LightState_t s)

{

uint8_t tx[4] = {FRAME_START, BROADCAST_ADDR, CMD_SET_STATE, (uint8_t)s};

// 发送指示

HAL_GPIO_WritePin(GPIOA, DEBUG_TX_PIN, GPIO_PIN_SET);

// 切换到发送模式

HAL_GPIO_WritePin(SET_GPIO_Port, SET_Pin, GPIO_PIN_SET);

HAL_Delay(1);

// 发送数据

HAL_UART_Transmit(&huart1, tx, 4, 100);

tx_count++;

// 等待发送完成

while(__HAL_UART_GET_FLAG(&huart1, UART_FLAG_TC) == RESET);

HAL_Delay(1);

// 切换回接收模式

HAL_GPIO_WritePin(SET_GPIO_Port, SET_Pin, GPIO_PIN_RESET);

// 发送指示关闭

HAL_Delay(10);

HAL_GPIO_WritePin(GPIOA, DEBUG_TX_PIN, GPIO_PIN_RESET);

}

/**

* @brief 处理接收到的数据帧（在主循环中调用）

* @retval None

*/

void ProcessReceivedFrame(void)

{

uint8_t* f = rx_buf;

// 验证帧头

if(f[0] != FRAME_START)

{

return;

}

// 验证目标地址：广播或本机ID

uint8_t target_addr = f[1];

if(target_addr != BROADCAST_ADDR && target_addr != MY_ID)

{

return;

}

// 验证命令

if(f[2] != CMD_SET_STATE)

{

return;

}

// 获取新状态

uint8_t new_state = f[3];

// 验证状态值是否有效

if(new_state <= STATE_BLUE)

{

// 更新状态

state = (LightState_t)new_state;

UpdateLEDs();

rx_count++;

// 设置闪烁标志

need_blink = 1;

}

}

/* USER CODE END 4 */

void Error_Handler(void)

{

// 不要关闭全局中断！

while(1)

{

// 错误指示：快速闪烁DEBUG_TX_PIN

HAL_GPIO_TogglePin(GPIOA, DEBUG_TX_PIN);

HAL_Delay(50);

}

}